2MP IMX307 Low Light 120dB IP Camera Module

- Product features

- &H.265/H.264/MJPEG

- &Video resolution：1920x1080/30fps

- &Motor Lens: 2.8-12mm/6-22mm/5-50mm（optional）

- &True WDR 120DB and AI denoise function

- &Camera module built-in RTC battery

&support 2ch audio in&1ch audio out,1ch alarm in,1ch reset.

- &TF card storage & alarm out &POE&WIFI are optional

- &Support ONVIF Profile S

| Order Model: M307H16V60-L11 |  |  |
| --- | --- | --- |
| CPU Chips |  | HI3516CV610-00S |
| DDR |  | 4Gbit DDR3 |
| Flash |  | 2Gbit SPI NAND Flash |
| Module Size: |  |  |
| One board | PCB size :38mm*38mm， Lens holder size:34mm*34mm |  |
| Image |  |  |
| CMOS Sensor |  | SONY IMX307，1/2.8Inch Progressive Scan CMOS sensor |
| Wide dynamic range |  | >120dB |
| Motor lens |  | 2.8-12mm/6-22mm/5-50mm（(optional） |
| Video resolution |  | 1920*1080P/30fps |
| Video coding |  | H.265/H.264/MJPEG |
| Stream |  | Main stream：1920*1080 1280*720 1-30fps（Optional 60fps） Sub stream：1280*720 704*576 352*288 1-30fps（Optional 60fps） |
| Video Bit rate |  | 16kbps~20Mbps，CBR/VBR/ |
| S/N Ratio |  | ≥58dB |
| Min Illumination |  | Color 0.01Lux@F1.2 |
| Audio |  |  |
| Audio Interface |  | 1ch MIC In&Line In 1ch Line out(support voice intercom) |
| Audio Format |  | G.711A、G.711U |
| Function |  |  |
| AE |  | support |
| AGC |  | support |
| AWB |  | support |
| 3D |  | support |
| Privacy Mask |  | Support 4 areas |
| ROI |  | support |
| Off network record video |  | support |
| Motion detect |  | Support 4detect areas |
| Day&Night Switch |  | Built-in intelligent algorithm to auto control IR CUT switch,color and white&night auto switch |
| Video adjust |  | Adjustable image contrast, brightness, saturation, sharpness and image mirroring, flipping, fog penetration, strong light suppression, backlight compensation, noise reduction level, digital wide dynamic, etc. |
| Corridor mode |  | support |
| Remote manager |  | System setting/Video stream management/Account management/Network management |
| Alarm Detect |  | E-mail alarm,PC client alarm,Motion detect alarm |
| Function Interface |  | 1ch Reset |
|  |  | 2group TTL serial port,use to connect communicate device or PTZ control (1 ch can be choose RS485 port) |
|  |  | 1ch alarm in(standard),1ch alarm out（optional,need add one board） |
|  |  | SD/TF card storage，max 1T（optional,need add one board） |
| OSD |  | OSD place ,color ,words can be config |
| Network |  |  |
| Ethernet Interface |  | 10M/100M RJ45 Network Interface |
| Video stream |  | Dual stream（Three stream to be customized） |
| Network protocol |  | TCP/IP、UDP、RTP、RTSP、RTCP、RTMP、HTTP、DNS、DDNS、DHCP、NTP、PPPOE、SMTP、UPNP; |
| ONVIF |  | ONVIF Profile S |
| CGI |  | support |
| Environment |  |  |
| Working temperature |  | -20°~+70° |
| Working humidity |  | 10%~90% |

Module pin definition: