2MP IMX662 Start Light 120dB IP Camera Module

- Product features

- &H.265/H.264/MJPEG

- &Video resolution：1920x1080/30fps

- &Motor Lens: 2.8-12mm/6-22mm/5-50mm（optional）

- &True WDR 120DB and AI denoise function

- &Camera module built-in RTC battery

&support 2ch audio in&1ch audio out,1ch alarm in,1ch reset.

- &TF card storage & alarm out &POE&WIFI are optional

- &Support ONVIF Profile S

| Order Model: M662H16V62-L11 |  |  |  |
| --- | --- | --- | --- |
| CPU Chips |  | HI3516CV610-20S |  |
| Flash |  | 2Gbit SPI NAND Flash |  |
| Module Size: |  |  |  |
| One board | PCB size :38mm*38mm， Lens holder size:34mm*34mm |  |  |
| Image |  |  |  |
| CMOS Sensor |  | SONY IMX662，1/2.8 Inch Progressive Scan CMOS sensor |  |
| Wide dynamic range |  | >120dB |  |
| Motor lens |  | 2.8-12mm/6-22mm/5-50mm（optional） |  |
| Video resolution |  | 1920*1080P/30fps |  |
| Video coding |  | H.265、H.264、 MJPEG |  |
| Stream |  | Main stream：1920*1080 1280*720 1-30fps（Optional 60fps） |  |
|  |  | Sub stream：1280*720 704*576 352*288 1-30fps（Optional 60fps） |  |
| Video Bit rate |  | 16kbps~20Mbps，CBR/VBR |  |
| S/N Ratio |  | ≥58dB |  |
| Min Illumination |  | Color 0.005Lux@F1.2 |  |
| Audio |  |  |  |
| Audio Interface |  | 1ch MIC In&Line In 1ch Line out (support voice intercom) |  |
| Audio Format |  | G.711A、G.711U |  |
| Function |  |  |  |
| AE |  | support |  |
| AGC |  | support |  |
| AWB |  | support |  |
| 3D |  | support |  |
| Privacy Mask |  | Support 4 areas |  |
| ROI |  | support |  |
| Off network record video |  | support |  |
| Motion detect |  | Support 4detect areas |  |
| Day&Night Switch |  | Built-in intelligent algorithm to auto control IR CUT switch,color and white&night auto switch |  |
| Video adjust |  | Adjustable image contrast, brightness, saturation, sharpness and image mirroring, flipping, fog penetration, strong light suppression, backlight compensation, noise reduction level, digital wide dynamic, etc. |  |
| Corridor mode |  | support |  |
| Remote manager |  | System setting/Video stream management/Account management/Network management |  |
| Alarm Detect |  | E-mail alarm,PC client alarm,Motion detect alarm |  |
| Function Interface |  | 1ch Reset |  |
|  |  | 2group TTL serial port,use to connect communicate device or PTZ control (1 ch can be choose RS485 port) |  |
|  |  | 1group I²C interface |  |
|  |  | 1ch alarm in(standard),1ch alarm out（optional,need add one board） |  |
|  |  | SD/TF card storage，max 1T（optional,need add one board） |  |
|  |  | USB interface UVC protocol video out; MJPEG /H264 coding， support 1080P、720P、VGA etc.(Optional) |  |
| OSD |  | OSD place ,color ,words can be config |  |
| Network |  |  |  |
| Ethernet Interface |  |  | 10M/100M RJ45 Network Interface |
| WIFI |  | Support (optional) |  |
| Video stream |  | Dual stream（Three stream to be customized） |  |
| Network protocol |  | Support TCP/IP、UDP、RTP、RTSP、RTCP、RTMP、HTTP、DNS、DDNS、DHCP、NTP、PPPOE、SMTP、UPNP; |  |
| ONVIF |  | Support ONVIF ProfileS |  |
| GB/T28181 |  | Support |  |
| CGI |  | Support |  |
| Environment |  |  |  |
| Working temperature |  |  | -20°~+70° |
| Working humidity |  |  | 10%~90% |

Module pin definition: