5MP IMX675 Starlight AI IP Camera Module

- Product features

- &H.265/H.264/MJPEG

- &Video resolution：2592*1944/25fps

- &Motor Lens: 2.8-12mm/6-22mm/5-50mm（optional）

- &True WDR 120DB and AI denoise function

- &Camera module built-in RTC battery

&support 2ch audio in&1ch audio out,1ch alarm in,1ch reset.

- &TF card storage & alarm out &POE&WIFI are optional

- &Support ONVIF Profile S

| Order Model: M675H16V60-L11 |  |  |  |
| --- | --- | --- | --- |
| CPU Chips |  |  | HI3516CV610-00S |
| DDR |  |  | 4Gbit DDR3 |
| Flash |  |  | 2Gbit SPI NAND Flash |
| Module Size: |  |  |  |
| One board |  | PCB size :38mm*38mm， Lens holder size:34mm*34mm |  |
| Image |  |  |  |
| CMOS Sensor |  |  | SONY IMX675，1/2.8″Progressive Scan CMOS |
| Motor lens |  |  | 2.8-12mm/6-22mm/5-50mm（(optional） |
| Video resolution |  |  | - 2592*1944/25fps |
| Video coding |  |  | H.265/H.264/MJPEG |
| Video | Main stream |  | 2592*1944 2560*1440 1920*1080 1-30fps |
|  | Sub stream |  | 1920*1080 1280*720 704*576 640*480 352*288 1-30fps |
| Video Bit rate |  |  | 32kbps~80Mbps，CBR/VBR |
| S/N Ratio |  |  | ≥54dB |
| Min Illumination |  |  | Color 0.005Lux@F1.2 |
| Audio |  |  |  |
| Audio Interface |  |  | 1ch audio in(1ch Line In &1ch MIC In) 1ch Line out,support voice intercom |
| Audio compression |  |  | G.711A、G.711U |
| Image |  |  |  |
| AE |  |  | Support |
| AGC |  |  | Support |
| AWB |  |  | Support |
| 3D |  |  | Support |
| Privacy Mask |  |  | Up to set 4 Privacy mask area |
| ROI |  |  | Support |
| Off network record video |  |  | Support |
| Motion detect |  |  | Up to set 4 motion detect area |
| Day&Night Switch |  |  | Built-in intelligent algorithm to auto control IR CUT switch,color and white&night auto switch |
| Video adjust |  |  | Adjustable image contrast, brightness, saturation, sharpness and image mirroring, flipping, fog penetration, strong light suppression, backlight compensation, noise reduction level, digital wide dynamic, etc. |
| Corridor mode |  |  | support |
| Remote manager |  |  | System setting/Video stream management/Account management/Network management |
| Alarm Detect |  |  | E-mail alarm,PC client alarm,Motion detect alarm |
| Function Interface |  |  | 1ch Reset |
|  |  |  | 2group TTL serial port,use to connect communicate device or PTZ control(1 ch can be choose RS485 port) |
|  |  |  | 1ch alarm in(standard),1ch alarm out（optional,need add one board） |
|  |  |  | SD/TF card storage，max 128G（optional,need add one board） |
|  |  |  | - 1ch USB,support UVC protocol video output - support MJPEG/H264 Coding,5MP/2K/1080P/720P/D1/VGA resolution optional |
| OSD |  |  | OSD place ,color ,words can be config |
| Network |  |  |  |
| Ethernet Interface |  |  | 10M/100M RJ45 network interface |
| Video stream |  |  | Dual stream（Three stream to be customized） |
| Network protocol |  |  | TCP/IP、UDP、RTP、RTSP、RTCP、RTMP、HTTP、DNS、DDNS、DHCP、NTP、PPPOE、SMTP、UPNP; |
| ONVIF |  |  | ONVIF ProfileS |
| GB/T28181 |  |  | support |
| CGI |  |  | support |
| Environment |  |  |  |
| Working temperature |  |  | -20°~+70° |
| Working humidity |  |  | 10%~90% |

Module pin definition: